Q&D Check I/O Ports v1.1 by PhG

Syntax : CHKPORTS <[$]port [[$]count]]|-free|-used|-map> [option]...

This program checks whether a port address ([$0000..$03ff]) is free or not.

Even though valid range for post scanning is [$0000..$03ff],

note [$0000..$01FF] range should be avoided (or tested with extreme caution).

Note, too, that port reading MAY hang computer : BEWARE and BE CAUTIOUS !

-free    show possibly free ports map in the [$0200..$03ff] range

-used    show possibly used ports map in the [$0200..$03ff] range

-map     show merged -free and -used maps

-m#      scan method (defaut is 1)

1 = word @ port                -- free if value is $ffff

2 = word @ port, word @ port   -- free if values are different

3 = byte @ port, byte @ port   -- free if values are $ff

4 = byte @ port, byte @ port   -- free if values are different

5 = byte @ port, byte @ port+1 -- free is values are $ff

6 = byte @ port, byte @ port+1 -- free if values are different

-w#      map display width ({1,2,4}, default is 4)

-c:$     map character set (untested,free,used,skipped), default is "_.#!"

-unsafe  allow program to scan dangerous ports :

-- [$0320..$032f] (hard disk controller)

-- [$03f0..$03f7] (floppy disk controller)

WARNING ! DATA LOSS MAY HAPPEN IF THESE PORTS ARE SCANNED !

Examples : CHKPORTS $220 16

CHKPORTS -free -w2